PHL7000 SDK User Guide for embedded visual C++ 1.0.6.3

| Date | Revision | Revised | Changes |
| --- | --- | --- | --- |
| 11/06/2006 | 1.0.0.0 | Luke Li | First release SDK for PHL7000 |
| 12/15/2006 | 1.0.3.0 | Luke Li | Keypad backlight function modified |
| 1/10/2007 | 1.0.5.0 | Luke Li | Add function BCT_SendSerial |
| 3/7/2007 | 1.0.6.0 | Luke Li | Fixed the problem that Readtime Command doesn't work when using SendSerial function. |
| 7/6/2007 | 1.0.6.1 | Luke Li | Add function BCT_GetConfigurationNumber BCT_GetSerialNumber BCT_MIC_SET BCT_MIC_GET |
| 7/6/2007 | 1.0.6.2 | Luke Li | Fixed the Barcode function can not run in OS version 2/1/2008 or later version. |
| 9/4/2008 | 1.0.6.3 | Luke Li | Fix the ASCII code “NULL” will be ignored. |
| 10/5/2008 | 1.0.7.0 | Luke Li | Add function Set_INPUT_MODE Get_INPUT_MODE |

Table of Contents

	BCT_Send_CMD	4

	BCT_OnOff_SDK	5

	BCT_BarCodeScan	6

	BCT_GetBarCode	7

	BCT_ EnableKeyNotification	8

	BCT_ DisableKeyNotification	9

	BCT_ MaskBarCodeKeyWEDGE	10

	BCT_ UnMaskBarCodeKeyWEDGE	11

	BCT_ ShowVersion	12

	BCT_ GetConfigurationNumber	13

	BCT_ GetSerialNumber	14

	BCT_ LCDSetBacklight	15

	BCT_ LCDGetBacklight	16

	BCT_ KPDSetBacklight	17

	BCT_KPDGetBacklight	18

	BCT_MIC_SET	19

	BCT_MIC_GET	20

- BCT_Send_CMD

Description:

We can send command to OPTICON barcode scanner via this function.

Function Prototype:

BOOL BCT_Send_CMD(BYTE* wrCMD)

Parameters:

wrCMD:[Input]

the pointer to a buffer sending command to MDL/MDI.

Return Values:

Nonzero (TRUE) indicates success.

Zero (FALSE) indicates failure

Requirements:

Header: OBCSDKEVC.h

Library: OBCSDKEVC.Lib

- BCT_OnOff_SDK

Description:

We can enable/disable default BCT when we are using own develop BCT.

Function Prototype:

BOOL BCT_OnOff_SDK(BOOL Flag)

Parameters:

Flag:[Input]

TRUE : enable to use SDK for developing the tools and disable the default BCT.

FALSE: disable to use SDK for developing the tools and enable the default BCT.

Return Values:

Nonzero (TRUE) indicates success.

Zero (FALSE) indicates failure

Requirements:

Header: OBCSDKEVC.h

Library: OBCSDKEVC.Lib

- BCT_BarCodeScan

Description:

This function triggers the MDL/MDI scanner to start the scanning.

Function Prototype:

BOOL BCT_BarCodeScan(void)

Parameters: None

Return Values:

Nonzero (TRUE) indicates success.

Zero (FALSE) indicates failure

Requirements:

Header: OBCSDKEVC.h

Library: OBCSDKEVC.Lib

- BCT_GetBarCode

Description:

When the scanning process is interrupted (got the data or force stop), system sends the WM_BCS_NOTIFY message to every window (except Child-Window).

This function can gets the scanning status.

Function Prototype:

BOOL BCT_GetBarCode(BYTE* BarCode,

PINT pScanstatus)

Parameters:

BarCode: [Output]

The point to a buff reserve the scanning data

pScanstatus: [Output]

The point to a buff reserve scanning status .We can get the scanning data when status is BarCodeRead_OK.

pScanstatus

BarCodeRead_OK

BarCodeRead_Timeout

BarCodeRead_ NotReady

BarCodeRead_ Disable

BarCodeRead_ ForceStop

Return Values:

Nonzero (TRUE) indicates success.

Zero (FALSE) indicates failure

Requirements:

Header: OBCSDKEVC.h

Library: OBCSDKEVC.Lib

- BCT_ EnableKeyNotification

Description:

This function enables 3 kinds of function keys.

Function Prototype:

BOOL BCT_EnableKeyNotification(BYTE KeyMask)

Parameters:

| KeyMask | Description |
| --- | --- |
| Ctl_ScanKey_Notify | Enable Scan Key |
| Ctl_FnKey_Notify | Enable Fn Key |
| Ctl_AlphaKey_Notify | Enable Alpha Key(Shift Key) |

Return Values:

Nonzero (TRUE) indicates success.

Zero (FALSE) indicates failure

Requirements:

Header: OBCSDKEVC.h

Library: OBCSDKEVC.Lib

- BCT_ DisableKeyNotification

Description:

This function disables 3 kinds of function keys, and function keys will only return KEY DOWN/KEY UP message.

Function Prototype:

BOOL BCT_DisableKeyNotification(BYTE KeyMask)

Parameters:

| KeyMask | Description |
| --- | --- |
| Ctl_ScanKey_Notify | Disable Scan Key |
| Ctl_FnKey_Notify | Disable Fn Key |
| Ctl_AlphaKey_Notify | Disable Alpha Key(Shift Key) |

Return Values:

Nonzero (TRUE) indicates success.

Zero (FALSE) indicates failure

Requirements:

Header: OBCSDKEVC.h

Library: OBCSDKEVC.Lib

- BCT_ MaskBarCodeKeyWEDGE

Description:

This function makes Keyboard Message to be “Disable”.

Function Prototype:

BOOL BCT_MaskBarCodeKeyWEDGE (void)

Parameters: None

Return Values:

Nonzero (TRUE) indicates success.

Zero (FALSE) indicates failure

Requirements:

Header: OBCSDKEVC.h

Library: OBCSDKEVC.Lib

- BCT_ UnMaskBarCodeKeyWEDGE

Description:

This function makes Keyboard Message to be “Type Writing”.

Function Prototype:

BOOL BCT_UnMaskBarCodeKeyWEDGE (void)

Parameters: None

Return Values:

Nonzero (TRUE) indicates success.

Zero (FALSE) indicates failure

Requirements:

Header: OBCSDKEVC.h

Library: OBCSDKEVC.Lib

- BCT_ ShowVersion

Description:

This function can get MDL/MDI firmware version.

Function Prototype:

BOOL BCT_ShowVersion (LPTSTR SwVerStr)

Parameters:

SwVerStr:[OUTPUT]

the pointer to a buffer receiving firmware version.

Return Values:

Nonzero (TRUE) indicates success.

Zero (FALSE) indicates failure

Requirements:

Header: OBCSDKEVC.h

Library: OBCSDKEVC.Lib

- BCT_ GetConfigurationNumber

Description:

This function can get Configuration Number.

Function Prototype:

BOOL BCT_GetConfigurationNumber (char* buffer1)

Parameters:

buffer1:[OUTPUT]

the pointer to a buffer receiving Configuration Number.

Return Values:

Nonzero (TRUE) indicates success.

Zero (FALSE) indicates failure

Requirements:

Header: OBCSDKEVC.h

Library: OBCSDKEVC.Lib

- BCT_ GetSerialNumber

Description:

This function can get Serial Number.

Function Prototype:

BOOL BCT_GetSerialNumber (char* buffer1)

Parameters:

buffer1:[OUTPUT]

the pointer to a buffer receiving Serial Number.

Return Values:

Nonzero (TRUE) indicates success.

Zero (FALSE) indicates failure

Requirements:

Header: OBCSDKEVC.h

Library: OBCSDKEVC.Lib

- BCT_ LCDSetBacklight

Description:

This function can set LCD backlight status.

Function Prototype:

BOOL BCT_LCDSetBacklight (int Intensity)

Parameters:

Intensity:[INPUT]

The value range of Intensity is from 0 to and including 6.

Return Values:

Nonzero (TRUE) indicates success.

Zero (FALSE) indicates failure

Requirements:

Header: OBCSDKEVC.h

Library: OBCSDKEVC.Lib

- BCT_ LCDGetBacklight

Description:

This function can get LCD backlight status.

Function Prototype:

int BCT_LCDGetBacklight (int Intensity)

Parameters: NONE.

Return Values:

The status of LCD backlight. It’s from 0 to 6.

Requirements:

Header: OBCSDKEVC.h

Library: OBCSDKEVC.Lib

- BCT_ KPDSetBacklight

Description:

This function can set Keyboard backlight status.

Function Prototype:

void BCT_KPDSetBacklight (BOOL fLight)

Parameters:

fLight:[INPUT]

TRUE: enable Keyboard backlight.

FASLE: disable keyboard backlight.

Return Values:NONE.

Requirements:

Header: OBCSDKEVC.h

Library: OBCSDKEVC.Lib

- BCT_KPDGetBacklight

Description:

This function can get keyboard backlight status.

Function Prototype:

int BCT_LCDGetBacklight (void)

Parameters: NONE.

Return Values:

The status of keyboard backlight.

1 indicates keyboard backlight is lighting.

0 indicates keyboard backlight is dark.

Requirements:

Header: OBCSDKEVC.h

Library: OBCSDKEVC.Lib

- BCT_MIC_SET

Description:

This function can set volume of Microphone.

Function Prototype:

void BCT_MIC_SET (ushort usGainLv)

Parameters:

usGainLv:[INPUT]

The value range of usGainLv is from 0 to and including 255.

Return Values: NONE.

Requirements:

Header: OBCSDKEVC.h

Library: OBCSDKEVC.Lib

- BCT_MIC_GET

Description:

This function can get volume of Microphone.

Function Prototype:

ushort BCT_MIC_GET (void)

Parameters: NONE.

Return Values:

The value range is from 0 to and including 255

Requirements:

Header: OBCSDKEVC.h

Library: OBCSDKEVC.Lib

- Set_INPUT_MODE

Description:

This function can set Input mode.

Function Prototype:

BOOL BCT_MIC_GET (BYTE InputMode)

Parameters:

InputMode : [INPUT]

0: Normal Mode.

1: Alpha Mode (PHL7000 only)

2: Fn Mode

Return Values:

Nonzero (TRUE) indicates success.

Zero (FALSE) indicates failure

Requirements:

Header: OBCSDKEVC.h

Library: OBCSDKEVC.Lib

- Get_INPUT_MODE

Description:

This function can get Input mode status.

Function Prototype:

BYTE BCT_MIC_GET (void)

Parameters:

None.

Return Values:

0: Normal Mode.

1: Alpha Mode. (PHL7000 only)

2: Fn Mode.

Requirements:

Header: OBCSDKEVC.h

Library: OBCSDKEVC.Lib